Readme information for the FY-3H/HIRAS-II Spectral Response Function (SRF) Dataset for public release

This document provides usage instructions for the Spectral Response Function (SRF) of the HIRAS-II instrument onboard the FY-3H satellite. HIRAS-II is a Fourier Transform Infrared (FTIR) hyperspectral interferometer, which performs spectral separation through a Michelson interferometer.

For an interferometric hyperspectral instrument, the spectral resolution is determined by the maximum optical path difference of the moving mirror. In the case of HIRAS-II, the same sampling interval is applied to all three spectral bands—Longwave Infrared (LWIR), Midwave Infrared 1 (MWIR1), and Midwave Infrared 2 (MWIR2)—and thus, the three bands have the same spectral resolution.

The SRF of HIRAS-II follows a Sinc (Cardinal Sine) function, which is inherently determined by the instrument’s characteristics, and is identical across all spectral channels. The SRF file consists of two columns of data: the first column contains the wavenumber (cm⁻¹), while the second column represents the corresponding spectral response. The wavenumber values are defined on an evenly spaced grid ranging from –15 cm⁻¹ to +15 cm⁻¹, symmetric around zero. This represents the spectral response within the neighboring wavenumber range of –15 cm⁻¹ to +15 cm⁻¹ relative to the central wavenumber of a given channel. In other words, for a spectral channel centered at λ, the SRF describes the response over the range (λ – 15, λ + 15). The spectral response values are normalized such that the maximum response equals 1, and at zero offset, the SRF value is exactly 1.

When using HIRAS-II Level 1 (L1) data in practice, it is recommended to apply a Hamming apodization function to suppress the side lobes of the Sinc function. However, this process broadens the spectral resolution of the L1 data and alters the SRF, which will no longer maintain a pure Sinc form.