Readme information for the FY-3H/MERSI-III

Spectral Response Function (SRF) Dataset for public release

This document provides instructions for the use of the Spectral Response Function (SRF) of the Medium Resolution Spectral Imager III (MERSI-III) onboard the FY-3H satellite. The MERSI-III is one of the primary payloads on the FengYun-3H (FY-3H) satellite. Building upon the MERSI-II instrument flown on the FY-3D satellite, it has been further improved, focusing on enhancing on-orbit radiometric calibration accuracy, detection sensitivity, and operational lifetime. The instrument comprises 25 spectral channels: 16 visible and near-infrared (VNIR) channels, 3 shortwave infrared (SWIR) channels, and 6 mid- to long-wave infrared (MIR/LWIR) channels. Among these 25 channels, six have a ground resolution of 250 meters and 19 have a ground resolution of 1000 meters.

The MERSI-III SRFs are normalized quasi-Gaussian functions, derived from monochromator-based laboratory measurements. Minor variations in SRF exist among different detector pixels within the same channel. The SRF data file contains two columns: the first column lists spectral wavelengths in nanometers (nm), and the second column provides the corresponding spectral response values. The spectral wavelengths are given as an equidistant series with a step size of approximately 0.5 nm, covering a spectral range from 300 nm to 1200 nm. For a given spectral channel λ, the spectral response values are normalized relative to the peak response, such that the SRF reaches its maximum value of 1 at the central wavelength of the channel.

Table 1 Channel characteristics of MERSI-III sensor

| 波段 | 中心波长 (m) | 带宽(nm) |
| --- | --- | --- |
| 1 | 0.47 | 50 |
| 2 | 0.55 | 50 |
| 3 | 0.65 | 50 |
| 4 | 0.865 | 50 |
| 5 | 1.38 | 30 |
| 6 | 1.64 | 50 |
| 7 | 2.13 | 50 |
| 8 | 0.412 | 20 |
| 9 | 0.443 | 20 |
| 10 | 0.49 | 20 |
| 11 | 0.555 | 20 |
| 12 | 0.67 | 20 |
| 13 | 0.709 | 20 |
| 14 | 0.746 | 20 |
| 15 | 0.865 | 20 |
| 16 | 0.905 | 20 |
| 17 | 0.936 | 20 |
| 18 | 0.94 | 50 |
| 19 | 1.03 | 20 |
| 20 | 3.8 | 180 |
| 21 | 4.05 | 155 |
| 22 | 7.2 | 500 |
| 23 | 8.55 | 300 |
| 24 | 10.8 | 1000 |
| 25 | 12.0 | 1000 |