Readme information for the FY-3H/MWRI-II

Spectral Response Function (SRF) Dataset for public release

Throughout this readme, the reader is assumed to have basic familiarity with MWRI and microwave radiometer design. The FY-3H/MWRI-II Spectral Response Functions (SRFs) provide the exact bandpass response within 3-dB bandwidth. The data is arranged in different excel sheet with different symbols (see Table 1). In each sheet, for single passband channel, there are 2 columns: band RF frequency (Hz), magnitude (dB). For double passbands, there are 3 columns: lower band RF frequency (Hz), Upper band RF frequency (Hz) and magnitude (dB).

Table 1 Channel characteristics of MWRI-II sensor

| No. | Center frequency（GHz） | Polarization | symbol |
| --- | --- | --- | --- |
| 1 | 10.65 | H | “10H” |
| 2 | 10.65 | V | “10V” |
| 3 | 18.7 | H | “18H” |
| 4 | 18.7 | V | “18V” |
| 5 | 23.8 | H | “23H” |
| 6 | 23.8 | V | “23V” |
| 7 | 36.5 | H | “36H” |
| 8 | 36.5 | V | “36V” |
| 9 | 50.30 | H | “54H-1” |
| 10 | 50.30 | V | “54V-1” |
| 11 | 52.61 | H | “54H-2” |
| 12 | 52.61 | V | “54V-2” |
| 13 | 53.24 | H | “54H-3” |
| 14 | 53.24 | V | “54V-3” |
| 15 | 53.75 | H | “54H-4” |
| 16 | 53.75 | V | “54V-4” |
| 17 | 89.0 | H | “89H” |
| 18 | 89.0 | V | “89V” |
| 19 | 118.7503±3.2 | V | “118-1” |
| 20 | 118.7503±2.1 | V | “118-2” |
| 21 | 118.7503±1.4 | V | “118-3” |
| 22 | 118.7503±1.2 | V | “118-4” |